GNS3 images

Download ASA image

https://drive.google.com/file/d/1JdEjOlGXxBjiRZbbrQrXXAzzbvirtiRv/view

https://drive.google.com/file/d/18Ly20TlEW5Axk1irl57wYQxl0PCnAFk7/view

install steps here

https://www.youtube.com/watch?v=ueA_zFyyS48

Router image

c7200-adventerprisek9-mz.152-4.S7.image

https://drive.google.com/file/d/1ATASM6fOq8gSDYTtrIdoC0TttbazxIAY/view?usp=drive_link

# Router AS DHCP

R5 (config)# interface f0/0 R5 (config-if)# no shutdown R5 (config-if)# ip address 192.168.30.1 255.255.255.0 R5(config)#ip dhcp pool inside R5(dhcp-config)#network 192.168.30.0 255.255.255.0 R5(dhcp-config)#default-router 192.168.30.1 R5(dhcp-config)#domain-name learncisco.net R5(dhcp-config)#dns-server 192.168.30.89 192.168.30.90 R5 (config)# interface f0/1 R5 (config-if)# no shutdown R5 (config-if)# ip address 192.168.50.1 255.255.255.0 R5 (config)# interface f1/0 R5 (config-if)# no shutdown R5 (config-if)# ip address 192.168.40.2 255.255.255.0

VPC > ip dhcp

R5 # show ip dhcp binding

# R1 as DNS

R5 # configure terminal R5 (config)# ip dns server R5 (config)# ip domain lookup R5 (config)# ip name-server 192.168.30.1 R5(config)#ip domain-name cisco.com R5 (config)# ip host PC1. cisco.com 192.168.30.3 R5 (config)# ip host PC2 .cisco.com 192.168.30.4 R5 (config)# ip host PC3 .cisco.com 192.168.30.5 R5 (config)# ip host PC4 .cisco.com 192.168.30.6 R5 (config)# ip host PC5 .cisco.com 192.168.30.7 R5 (config)# ip host PC6 .cisco.com 192.168.30.8 R5 (config)# ip host PC7 .cisco.com 192.168.30.9 R5 (config)# ip host PC8 .cisco.com 192.168.30.10 R5 (config)# ip host PC9 .cisco.com 192.168.30.11 R5 (config)# ip host PC10 .cisco.com 192.168.30.12 R5 (config)# ip host printer .cisco.com 192.168.30.21 R5# show hosts R5 # copy running-config startup-config

# ASA configuration

! conf t ! interface GigabitEthernet0/1 nameif inside security-level 100 ip address 192.168.50.2 255.255.255.0 ! interface GigabitEthernet0/0 nameif outside security-level 0 ip address 192.168.10.5 255.255.255.0 !

! object network lan-corp subnet 192.168.30.0 255.255.255.0 nat (inside,outside) dynamic interface object network mgmt subnet 192.168.40.0 255.255.255.0 nat (inside,outside) dynamic interface ! ! route outside 0.0.0.0 0.0.0.0 192.168.10.1 route inside 192.168.40.0 255.255.255.0 192.168.50.1 route inside 192.168.30.0 255.255.255.0 192.168.50.1 ! policy-map global_policy class inspection_default inspect icmp !

access-list inside extended permit icmp any any access-list OUTSIDE extended permit icmp any any access-group inside in interface inside

### Step 4: Configure DHCP service on the ASA

dhcpd address 192.168.50.3-192.168.50.250 inside dhcpd dns 192.168.30.1 dhcpd lease 3600 dhcpd ping_timeout 50 dhcpd enable inside dhcprelay timeout 60

# IPSEC VPN configuration (R1 <> R3)

R1# show running config

R3 configuration

R3# show running config